Santa Rosa Swim Meet

Timing Equipment Documentation

Neptunes, High School North Bay League, Junior College and Masters

Updated 5/1/2023  by Chris Jones et. al  (707)799-5502 cell

North Bay League

This page intentionally left blank so page 1 can go in the binder sleeve.

Table of Contents

1.	Quantum	7

1.1.	Equipment Setup	7

Pool Configurations	7

Equipment list	10

Electrical Connections	12

Fixed Equipment Locations	14

Turn on equipment	17

Prepare for an outdoor environment	17

Prepare Printer	18

Adjust Volume of Starters	18

Test video scoreboard	18

1.2.	Quantum Software Configuration	19

Launch Software	19

Configure Timing	19

Create a new session	20

Configure Printer	21

Set Up Race Distances	22

Configure I/Os	23

Configure pool	25

Test start system	30

Test Touch Pads	31

Test Buttons	32

Configure Printing	33

Test Printer	34

Update Meet Schedule	35

1.3.	Timing Equipment Operator Duties	36

Project to Video Scoreboard	36

Open Race Timing Screen	38

Turn On Video Scoreboard for Results	38

Update Start List	39

Open Race Status Window	40

Prepare for Start	41

Verify Start	42

Edit Lanes During the Race	43

Store Results	44

1.4.	Intermediary Duties	45

2.	Dolphin	48

2.1.	Equipment Setup	48

2.2.	Timing Equipment Operator Duties	52

2.3.	Intermediary Duties	54

2.4.	Troubleshooting	57

3.	System 6	58

3.1.	Equipment Setup	58

3.2.	Timing Equipment Operator Duties	64

3.3.	Intermediary Duties	67

4.	Appendix	71

4.1.	Quantum Timing Equipment Operator Checklist	71

4.2.	Dolphin Timing Equipment Operator Checklist	73

4.3.	Colorado Timing Equipment Operator Checklist	75

4.4.	Intermediary Checklist	77

Table of Figures:

Figure 1: 8 Lanes Down the Pool Configuration.	7

Figure 2: 8 Lanes Across the Pool Configuration.	8

Figure 3: 10 Lanes Across the Pool Configuration.	9

Figure 4: Buried Deck Connector Locations.	14

Figure 5: Quantum Fixed Deck Plate Connections.	15

Figure 6: Fixed Harness and Scoreboard Connections.	16

Figure 7: Quantum Configuration Timing screen.	19

Figure 8: Quantum Add New Meet Screen.	20

Figure 9: Quantum Autoprint Screen.	21

Figure 10: Quantum Race Distance Table.	22

Figure 11: Quantum Race I/O Setup Screen.	23

Figure 12: Updated Quantum Race I/O Screen.	24

Figure 13: 8 Lanes Down the Pool Lane Picture.	25

Figure 14: 8 Lanes Down the Pool Equipment Picture.	26

Figure 15: Quantum 8 Lanes Down the Pool Configuration Screen.	27

Figure 16: Quantum 8 Lanes Across Configuration Screen.	28

Figure 17: Quantum 10 Lanes Across Configuration Screen.	29

Figure 18: Quantum Test Start Screen.	30

Figure 19: Quantum Touch Pad Test Screen.	31

Figure 20: Quantum Button Press Test Screen.	32

Figure 21: Quantum Initial Race Timing Screen.	33

Figure 22: Quantum Print On Paper Screen.	34

Figure 23: Quantum Import Schedule Screen.	35

Figure 24: Video Scoreboard control window.	37

Figure 25: Turn On Video Scoreboard for Results.	38

Figure 26: Quantum Refresh Start List Screen.	39

Figure 27: Quantum Open Race Ready Screen.	40

Figure 28: Quantum Prepared for Start screen.	41

Figure 29: Quantum Started Race Screen.	42

Figure 30: Quantum Lane Edit Menu.	43

Figure 31: Example Quantum marked up heat results printout.	46

Figure 32: Example Dolphin wireless stopwatch test results.	51

Figure 33: Example Dolphin results printout with missing time.	54

Figure 34: Example Dolphin results printout with more than .3 second span.	55

Figure 35: System 6 electrical connection diagram.	59

Figure 36: System 6 operator interface.	60

Figure 37: Example System 6 race results printout.	63

Figure 38: Example System 6 marked up heat results printout.	68

Table of Tables:

Table 1: Quantum timing equipment list.	11

Table 2: Quantum electrical connection list.	13

Table 3: Dolphin timing equipment list.	48

Table 4: Dolphin electrical connection list.	49

Table 5: System 6 timing equipment list.	58

Table 6: System 6 electrical connection list.	59

## Quantum

### Equipment Setup

#### Pool Configurations

8 lanes down the pool 25 yards:

Figure 1: 8 Lanes Down the Pool Configuration.

8 lanes down the pool 50 meters:

8 lanes across the pool:

Figure 2: 8 Lanes Across the Pool Configuration.

10 lanes across the pool configuration:

Figure 3: 10 Lanes Across the Pool Configuration.

#### Equipment list

| Item no. | Item | Note |
| --- | --- | --- |
| 1 | Timing laptop | With 2 USB A ports and ethernet port, with Quantum software installed and configured |
| 2 | Timing laptop power supply |  |
| 3 | Timing laptop USB-C to ethernet and USB-A adapter | Plug in to left side of laptop |
| 4 | Timing wireless mouse with USB adapter |  |
| 5 | Timing mouse pad |  |
| 6 | Timing printer | USB interface with toner and white printer paper installed |
| 7 | Timing printer power cord |  |
| 8 | Timing printer USB cable | USB-B to USB-A |
| 9 | Quantum start system #1 | For close end starts. Includes box with light, start in and start out ports mounted on tripod |
| 10 | Start system #1 power supply |  |
| 11 | Start microphone with start switch #1 |  |
| 12 | Start cable #1 |  |
| 13 | Quantum start system #2 | For far end starts such as 25 yard races. Includes box with light, loudspeaker, start in and start out ports placed on coaches table |
| 14 | Start system #2 power supply |  |
| 15 | Start microphone with start switch #2 |  |
| 16 | Start cable #2 |  |
| 17 | Quantum timing unit |  |
| 18 | Quantum timing unit power supply |  |
| 19 | Quantum timing unit USB cable | USB-B to USB-A |
| 20 | Quantum start cable |  |
| 21 | Quantum harness cable |  |
| 22-31 | 10 Quantum mobile harness boxes | For up to 10 lanes going across the pool |
| 32 | Quantum mobile harness terminator | For up to 10 lanes going across the pool |
| 33 | Quantum fixed harness | For 8 lanes going down the pool |
| 34-43 | 10 Quantum loudspeakers |  |
| 44 | Quantum mobile loudspeaker harness |  |
| 45-54 | 10 touchpads | Hang them on the pool gutter and tighten the clamps snugly so they don’t come loose and cause missed touches |
| 55-64 | 10 button 1s |  |
| 65 | Fixed 10-Lane video scoreboard |  |
| 66 | Video scoreboard laptop |  |
| 67 | Video scoreboard laptop power supply |  |
| 68 | Video scoreboard controller |  |
| 69 | Video scoreboard display |  |
| 70 | Video scoreboard display power supply |  |
| 71 | Video wireless mouse with USB adapter |  |
| 72 | Video mouse pad |  |
| 73 | Video controller HDMI cable |  |
| 74 | Video display HDMI cable |  |
| 75 | Video box serial cable |  |
| 76 | Video scoreboard ethernet cable |  |
| 77 | Fixed 10-lane numeric scoreboard |  |
| 78 | Numeric scoreboard cable |  |
| 79 | Power strip | At least 7 outlets for timing laptop, timing printer, timing unit, start system 1, video scoreboard controller, video laptop and video display |
| 80 | Extension cord 1 | For power strip |
| 81 | Extension cord 2 | For second start system |
| 82 | Computer room ethernet cable | Long enough to reach from timing laptop to computer room router or switch |
| 83 | Reliable power source with circuit breaker location known and accessible |  |

Table 1: Quantum timing equipment list.

#### Electrical Connections

| Connection No. | Item | From | To |
| --- | --- | --- | --- |
| 1 | Timing laptop power supply | Power strip | Timing laptop |
| 2 | Timing laptop USB-C to ethernet and USB-A adapter |  | Timing laptop USB-C left side |
| 3 | Timing wireless mouse USB adapter |  | Timing laptop |
| 4 | Timing printer power cord | Power strip | Timing printer |
| 5 | Timing printer USB cable | Timing printer | Timing laptop |
| 6 | Timing unit power supply 3 pin 12V | Power strip | Timing unit |
| 6 | Start system #1 power supply | Power strip | Starting system #1 External Power jack |
| 7 | Start microphone with start switch #1 |  | Starting system #1 e-Gun jack |
| 8 | Start cable #1 Amphenol | Start system #1 Start output | Timing unit start 1 input |
| 9 | Start system #2 power supply | Extension cord #2 | Starting system #2 External Power jack |
| 10 | Start microphone with start switch #2 |  | Starting system #2 e-Gun jack |
| 11 | Start cable #2 50 meter 4 pin reel black | Start system #2 Start output | Timing unit Start 2 input |
| 12-20 | For up to 10 lanes going across only: Mobile harness box 2 Mobile harness box 3 Mobile harness box 4 Mobile harness box 5 Mobile harness box 6 Mobile harness box 7 Mobile harness box 8 Mobile harness box 9 Mobile harness box 10 Mobile harness terminator |  | Mobile harness box 1 Mobile harness box 2 Mobile harness box 3 Mobile harness box 4 Mobile harness box 5 Mobile harness box 6 Mobile harness box 7 Mobile harness box 8 Mobile harness box 9 Mobile harness box 10 |
| 21 | Harness 1 cable 7 pin tan: For up to 10 lanes going across only | Mobile harness box 1 | Timing unit HA1 jack |
|  | For 8 lanes down the pool only | Fixed harness output box | Timing unit HA1 jack |
| 22-31 | For up to 10 lanes going across only: Loudspeakers 1-10 Loudspeaker harness 4 pin black |  | Loudspeaker harness Start system #1 Speaker out jack |
|  | For 8 lanes down the pool only: Loudspeakers 1-10 |  | Fixed harness starting block plate “S” jacks |
| 32-41 | For up to 10 lanes going across only: Touch pads 1-10 |  | Mobile Harness 1-10 “T” jacks |
|  | For 8 lanes down the pool only: Touch pads 1-10 |  | Fixed harness starting block plate “T” jacks |
| 42-51 | For up to 10 lanes going across only: Button 1 1-10 |  | Mobile Harness 1-10 “1” jacks |
|  | For 8 lanes down the pool only: Button 1 1-10 |  | Fixed harness starting block plate “1” jacks |
| 52 | Quantum timing unit USB cable | Quantum timing unit | Timing laptop USB-A right side |
| 53 | Video scoreboard laptop power supply | Power strip | Video laptop |
| 54 | Video scoreboard controller power cord |  | Power strip |
| 55 | Video scoreboard display power supply | Power strip | Video display |
| 56 | Video wireless mouse adapter |  | Video laptop |
| 57 | Video controller HDMI cable | Video laptop | Video controller |
| 58 | Video display HDMI cable | Video controller | Video display |
| 59 | Video controller DB9 serial cable | Timing unit Serial 1 DB9 output | Video controller Timer 1 input |
| 60 | Video scoreboard ethernet cable | Video controller Out 1 | Fixed Swiss Timing box LAN Port #1 |
| 61 | Numeric scoreboard serial cable | Timing unit Serial 2 DB9 numeric scoreboard output | Fixed Swiss Timing box Numeric scoreboard input jack Amphenol |
| 62 | Extension cord 1 | AC outlet #1 | Power strip |
| 63 | Extension cord 2 | AC outlet #2 | Start system #2 power supply |
| 64 | Computer room ethernet cable | Timing laptop USB-C to ethernet adapter ethernet port | Computer room router or switch |

Table 2: Quantum electrical connection list.

#### Fixed Equipment Locations

Deck Connectors

- Fixed deck plate lane connections to touch pad, button and speaker for 8 lanes down the pool only: #1 below (lane 8 shown)

- Fixed harness and scoreboard connections: #2 below

#2

#1

Figure 4: Buried Deck Connector Locations.

Deck Plate Details

For 8 lanes down the pool only

- Speaker “S”, #1 below

- Button “1”, #2 below

- Touch pad “T”, #3 below

#1

#3

#2

Figure 5: Quantum Fixed Deck Plate Connections.

Fixed Harness and Scoreboard Connections

- Harness “HA”, #1 below

- “Numeric” Scoreboard, #2 below

- “Speaker”, #3 below

- LAN port “1”, #4 below

#4

#3

#2

#1

Figure 6: Fixed Harness and Scoreboard Connections.

#### Turn on equipment

- Timing laptop

- Wireless mouse

- Timing printer

- Timing unit

- Start system #1

- Start system #2

- Video scoreboard

- Numeric scoreboard

- Video scoreboard laptop

- Video scoreboard controller

- Video scoreboard display

#### Prepare for an outdoor environment

- Clean timing laptop and video scoreboard displays with glass cleaner if dirty.

- Bring a sun shade to see the timing laptop display if the sun is at the wrong angle.

- Use paperweights to contain papers if windy.

- Bring flashlight if meet goes in to the night without adequate lighting.

#### Prepare Printer

If needed.  For club and masters meets only.  On timing laptop, launch Printers and Scanners.  Make sure connected printer is listed as default.  If not:

- Select connected printer

- Click Manage

- Click Set as Default

- Click X to close window

#### Adjust Volume of Starters

Do on both starters when 25 yard races are included in club meets.

- Press speaker button

- Select Internal or External speakers

- Adjust volume to avoid feedback

#### Test video scoreboard

See Timing Equipment Operator Duties section.

### Quantum Software Configuration

#### Launch Software

- If necessary, plug in USB dongle protection key that holds the Quantum license, if not hard coded on the laptop.

- Create session folder. Example “c:\Hy-Tek Meet Files\20230304 High School Invite”

- Launch Quantum AQ Swimming program.  This brings you to the Home page.

- Verify box in upper right is green, indicating that the Quantum-Aq Timer is connected to the PC via USB cable.  Be patient: this may take up to 10 seconds normally, and up to 1 minute the first time it makes the connection.

#### Configure Timing

- Click Configuration button.  This brings up the Timer tab.

- Click Timing tab.

- Verify Arming at start is set to 10 seconds not the 8 second default, shown in #1 below, to allow swimmers more time to get out of the pool.

#1

Figure 7: Quantum Configuration Timing screen.

#### Create a new session

A session is called a “meet” in Quantum but one Quantum “meet” is required per session within a meet.

- Click red Home button in upper right corner of screen.

- Click Race button.

- Press green “+” button to create a new meet, shown in #1 below.

#1

Figure 8: Quantum Add New Meet Screen.

- Type Meet name.  Do not use special characters.  Create one meet per session.  Example “20230304 High School Invite”. For multi-session meets indicate which session it is in the title.

- Click OK. This brings up the Race Settings Tab and Timing subtab.

#### Configure Printer

- Click Printer subtab.

- Select Autoprint on official, shown in #1 below, only if you are printing the results for each heat, such as for club, high school and Masters swimming.  Not needed for JC.

#1

Figure 9: Quantum Autoprint Screen.

#### Set Up Race Distances

- Click Tables subtab.

- Click Distances sub-subtab.

- Add any distances in the meet that are not listed in the table by clicking the green add arrow and filling the distance.  Notice uncommon Norcal Relay meet distances added below – 150, 250 and 300 yards.

#1

Figure 10: Quantum Race Distance Table.

Figure out how to switch from yards to meters and add document here.  Switching from default meters to yards as well as adding the uncommon race lengths included Omega setup both on-site and then subsequently remotely with a comment that they would have to do it again in the future, but I suggest they teach us how to fish instead.

#### Configure I/Os

- Click I/Os tab.

- Verify on the top row that SCB1 is set to Serial 1, SCB Swiss Timing Alpha alphanumeric jumbotron scoreboard and is selected, #1 below.

- Verify in the second row that SCB2 is set to Serial 2, SCB Calypso numeric scoreboard and is selected.

- Verify in the 4th row that the left column is Hy-Tek, 3rd column says DH Hytek and is selected.

- Verify the third row SCB3, 5th row DH2 and 6th row DH3 are all blank.

- Click 4th row second column to select Hytek file sharing folder.  This brings up a new window below.

- Click on “..” at right side of ePath for the files row.  A new folder navigation window pops up below.

- Navigate to folder specified by computer operator, such “c:\Hy=Tek Meet Files\20230304 High School Invite\” then click OK.

#6

#5

#4

#2

#1

#33

Figure 11: Quantum Race I/O Setup Screen.

- Verify ePath was updated to target file share folder, #1 below.

#1

Figure 12: Updated Quantum Race I/O Screen.

#### Configure pool

- Click Pool config tab

- 8 lanes down the pool:

Figure 13: 8 Lanes Down the Pool Lane Picture.

Figure 14: 8 Lanes Down the Pool Equipment Picture.

  - Toggle on Enable Changes button to be selected, #1 below.

  - Under Harnesses, select HA1 on right, and deselect 3 other options, #2 below.

  - Under Contacts, select touch pad and button 1 on right, and deselect other 8 options, #3 below.

  - Under Number of Lanes select 1-8, #4 below.

  - Under Lanes use recycle buttons and arrow keys as needed to get lane 1 on the top (far side of pool) and lane 8 on the bottom (close side of pool) in the right green column.  Ignore middle white and red left columns, #5 below.

  - Under General configuration, select Pool Length = 25 and Start/Finish Place Finish at Right, #6 below.

  - Click Save as Default, #7 below.

  - Toggle Enable Changes button to be deselected, #1 below.

#7

#6

#5

#4

#3

#2

#1

Figure 15: Quantum 8 Lanes Down the Pool Configuration Screen.

- 8 lanes across the pool:

Add photos of pool and equipment

  - Toggle on Enable Changes button to be selected, #1 below.

  - Under Harnesses, select HA1 on left, and deselect 3 other options, #2 below.

  - Under Contacts, select touch pad and button 1 on left, and deselect other 8 options, #3 below.

  - Under Number of Lanes select 1-8, #4 below.

  - Under Lanes use recycle buttons and arrow keys as needed to get lane 1 on the bottom (close side of pool) and lane 8 on the top (far side of pool) in the left green column.  Ignore middle white and red right columns, #5 below.

  - Under General configuration, select Pool Length = 25 and Start/Finish Place Finish at Left, #6 below.

  - Click Save as Default, #7 below.

  - Toggle Enable Changes button to be deselected, #1 below.

#7

#6

#4

#3

#2

#5

#1

Figure 16: Quantum 8 Lanes Across Configuration Screen.

Replace with proper screen shot

- 10 lanes across the pool:

Add photos of pool and equipment

  - Toggle on Enable Changes button to be selected, #1 below.

  - Under Harnesses, select HA1 on left, and deselect 3 other options, #2 below.

  - Under Contacts, select touch pad and button 1 on left, and deselect other 8 options, #3 below.

  - Under Number of Lanes select 1-10, #4 below.

  - Under Lanes use recycle buttons and arrow keys as needed to get lane 1 on the bottom (close side of pool) and lane 0 on the top (far side of pool) in the left green column; box 0 = lane 10.  Ignore middle white and red right columns, #5 below.

  - Under General configuration, select Pool Length = 25 and Start/Finish Place Finish at Left, #6 below.

  - Click Save as Default, #7 below.

  - Toggle Enable Changes button to be deselected, #1 below.

#4

#7

#6

#5

#3

#2

#1

Figure 17: Quantum 10 Lanes Across Configuration Screen.

Replace with proper screen shot

#### Test start system

- Click Pool test tab.

- Press starter e-Gun start button.  Make sure it goes beep and the clock starts going on the Quantum laptop, video scoreboard and numeric scoreboard, and that the laptop looks like this:

Figure 18: Quantum Test Start Screen.

#### Test Touch Pads

- Make sure each lane’s pad touch looks like this:

Figure 19: Quantum Touch Pad Test Screen.

#### Test Buttons

- Make sure each button press looks like this:

Figure 20: Quantum Button Press Test Screen.

#### Configure Printing

- Click Timing tab.  This brings up main run window with Timing tab open.

- Click Printer tab below the bottom box on the right, #1 below.

#1

Figure 21: Quantum Initial Race Timing Screen.

- Select Print On Paper if desired, #1 below.

#1

Figure 22: Quantum Print On Paper Screen.

#### Test Printer

Figure out how to do this

#### Update Meet Schedule

- Click on Schedule subtab, #1 below.

- Once the computer operator has downloaded the meet schedule to the Quantum computer, click on Hy-Tek, #2 below and select Import Meet Schedule.  This reads the quantum.sch file created by the Hy-Tek computer operator and populates the upper right table with events.  Verify that these events agree with the meet sheet.  Completion of this step confirms proper connection between the Quantum and Hy-Tek computers.

Figure out how to test getting times back to HyTek without making a practice meet, which defeats the purpose because changing meets can change the configuration and stop working.

#2

#1

Figure 23: Quantum Import Schedule Screen.

Update screen shot with the Hy-Tek menu dropped down showing Import schedule.

In the event of a problem connecting to the Hy-Tek computer, you can create a schedule manually, but no names will be displayed on the scoreboard.  See page 21 in Quantum user manual for instructions.

### Timing Equipment Operator Duties

#### Project to Video Scoreboard

Turn on video box first.

Project Pace Clock

Open Nova Studio app on video scoreboard laptop.

Bug: it always closes the first time.  Open it again.

Click Open icon.

Click Yes to confirm opening new item.

Select pace clock.

Click Open.

In Common Window 2, click Digital Clock.

Click Play button.

Verify time is projecting and is the correct time.

Project Video

Close Nova Studio app.

Open VACS_MSMP app.

It can take time to come up; do not re-double click it and have two instances running, it will not work.

Select File, Open.

Select 8 lanes or 210 lanes in two places.

Make sure all scoreboards are unchecked in the Swimming tab (for now), shown at #1 below.

Select Scoreboard 1 Event radio button, shown at #2 below.

Double click media player 1.tpl in the Scoreboard 1 column in the upper Sports Control box, shown in #3 below.

In the Media Player 1 box select Drone Video for background or American Flag New for the national anthem shown in #4 below.

Click Start shown in #5 below. Verify video is playing on the scoreboard.

To stop playing video click Stop.

Project Results

Need to close out app and restart app for each session.

Follow instructions for project video above except in the Sports Control box double click Start.tpl.

#5

#4

#3

#2

#1

Figure 24: Video Scoreboard control window.

#### Open Race Timing Screen

You will be running the equipment in the Timing Tab in Race Mode.  To navigate there:

  - In Quantum-AQ Swimming program, go to the Home screen by clicking the red house icon in the upper right corner.  The session should already be set up per the previous section.

  - Click Timing tab.

#### Turn On Video Scoreboard for Results

- Click SCB1 button, #1 below.

- Click On button, even though it says it is already on, #2 below.

#2

#1

Figure 25: Turn On Video Scoreboard for Results.

#### Update Start List

Do this once before the session for preseeded meets, and before each event for deck seeded meets.

- Click green HyTek button, #1 below.

- Click Refresh Start List Whole Meet, #2 below.  This reads the quantum.rec and quantum.slx file in the session folder populated by the computer seeding operator.

- Clock on event row, #3 below. That populates the heat window.

- Click on heat row, #4 below.  That populates the lower right swimmer window.

#4

#3

#2

#1

Figure 26: Quantum Refresh Start List Screen.

#### Open Race Status Window

- Select heat row, #1 below.

- Click Open, #2 below.  The race status window should appear, #3 below.

#1

#2

#1

Figure 27: Quantum Open Race Ready Screen.

During each heat:

#### Prepare for Start

- Make sure all data has been stored from previous heat; see Store Results below.

- Make sure Start Armed is checked, #1 below.

- Make sure all lanes selected are green, meaning the lane touch pads and buttons are detected, #2 below.

- If you are having trouble and need to hold the start, contact the starting referee or toggle the Start Armed button to disable their ability to start a race.

#2

#1

Figure 28: Quantum Prepared for Start screen.

#### Verify Start

Once race is started:

- Verify clock is running, #1 below.

- Watch the wall and listen for click sounds on the computer speaker when a touch is recorded.

- Watch the race window.  The yellow and red bars going across shows estimated progress of the swimmer.  If it goes all the way across, check to see if that swimmer missed a touch.

#1

Figure 29: Quantum Started Race Screen.

#### Edit Lanes During the Race

When there is an issue, right-click on green boxed lane number requiring editing, example lane 2 #1 below.  This brings up a pop-up window with some options, #2 below.

- Lap -1: select if an exiting swimmer in a relay exits after 10 seconds and touches the pad and adds an extra touch.

- Lap +1: select if the swimmer missed a touch.  Note: if there is a pattern on a lane, dispatch someone to troubleshoot.  Common causes of extra missed touches include:

  - Disconnected or broken touch pad wires

  - Pad clamps that come loose and the pad floats away from the edge

  - Not having clean and properly electrically greased touch pad banana plug connectors.

- Arm touchpad: select if there were extra touches that erroneously is reporting that the swimmer has finished and the swimmer is imminently going to finish.

- Disarm touchpad: not used.

- Edit Finish Time: if swimmer had a soft touch at the end, or if you missed adding touches during the race, and the button was pushed select this then type in the button time.  Why doesn’t Take Backup Time do this?

- DSQ: not used.

- DNS: select for no-shows.

- DNF: select if swimmer starts but does not finish a race.

- Take Backup Time: not used but would be if we can figure out how it works.

- Used/Unused: toggle to disable lanes for any athletes that scratched at the block.

#2

#1

Figure 30: Quantum Lane Edit Menu.

#### Store Results

- Verify no one is left swimming in the pool; failure to do so could lead to losing their results!

- When race is done, click Officialize thumbs up button, #1 below.  This automatically saves the results to the shared folder for Hy-Tek, and prints the results of the heat to the printer if selected.

- Click yellow right arrow icon to proceed to next heat or event in the schedule, #2 below.

#1

#2

### Intermediary Duties

This person sits between the Timing Equipment Operator and the Admin or head Referee.   This person’s job is to verify printer is working properly; troubleshoot and fix any printing problems that may occur while the timing equipment operator concentrates on capturing times, and collect all results paperwork and give it to the Admin for review.

After each race:

- Collect Race Summary Sheet from the printer.

- Circle the event number and heat number with a pencil.

- Verify button(s) or pad were pressed and the times did not span more than 0.3 seconds for each lane, and that the timing equipment selected the middle time as the Finish Time.

  - If so, draw a line next to the lane time column with a pencil indicating you have checked it and it is good.

  - If not, circle a single time if it is a single outlier or is missing, or circle the whole column if all the numbers are far from each other or all missing, to flag admin or head referee that there is a problem.

- If more than one button or pad was pushed and the span as greater than 0.3 seconds, verify that the timing equipment threw out the outlying data points.

- If one time was an outlier, make sure timing equipment picked the time of the two other times that was closest to the outlier for the Finish Time.

- If all three data points were spread far apart, make sure the timing equipment picked the middle time for the final time.

- If only one button was pushed, make sure the timing equipment selected it for the Finish Time.

- If there were no buttons pushed, or there was a problem with the timing system, notify the head Referee so they can look at the Lane Time Sheet or Coach’s Timer to determine the Finish Time.

- If a button or pad is regularly not being pressed but it appears like someone is trying to press it, check the finish cable harness seating and switch connection seating, or send someone over to talk to the timer and possibly swap out their button or a pad.  Note that this could cause a delay in the meet.

- If anyone was DQd after the event was over, write DQ in pencil on the Timing event program, tear off the carbon copy of the DQ form, and hand it to the Admin or Chief Stroke and Turn Judge.

- Compare order of finish on timing system printout to referee sheet as applicable.

Replace following picture with Quantum printout (CTS System 6 shown)

Figure 31: Example Quantum marked up heat results printout.

After each event:

Clip the paperwork together in the following order:

- DQ slips

- Colorado meet sheets

- Referee meet sheets

- Lane timer sheets

Give the stack of paperwork to the Runner who will bring it to the Computer Room.

## Dolphin

### Equipment Setup

Equipment list:

| Item no. | Item | Note |
| --- | --- | --- |
| 1 | Timing laptop | With 3 USB A ports and ethernet port, with Dolphin and Gadwin Printscreen software installed and configured |
| 2 | Timing laptop power supply |  |
| 3 | Wireless mouse with USB adapter |  |
| 4 | Mouse pad |  |
| 5 | Timing printer | USB interface with toner and white printer paper installed |
| 6 | Timing printer power cord |  |
| 7 | Timing printer USB cable | USB-B to USB-A |
| 8 | Start system | Includes box with speaker, light, start in and dual BNC start out ports mounted on tripod |
| 9 | Start system power supply |  |
| 10 | Start microphone with start switch |  |
| 11 | 1 Lane scoreboard |  |
| 12 | Scoreboard power cord |  |
| 13 | Dolphin base unit | Colorado Time Systems |
| 14 | Dolphin base unit USB cable | USB-B to USB-A |
| 15 | Dolphin starter unit | Mini-BNC output, make sure 9V battery is not low |
| 16 | Mini-BNC to BNC cable |  |
| 17 | BNC to dual banana adapter |  |
| 18 | Dolphin wireless scoreboard adapter |  |
| 19-42 | 24 Dolphin Wireless Stopwatches | Make sure batteries are not low |
| 43 | 25 spare fresh 9V batteries |  |
| 44 | Small flat blade screwdriver | To replace batteries |
| 45 | Power strip | At least 3 outlets for timing laptop, timing printer and starting system |
| 46 | Extension cord 1 | For power strip |
| 47 | Extension cord 2 | For scoreboard |
| 48 | Ethernet cable | Long enough to reach from timing laptop to computer room router or switch |
| 49 | Reliable power source with circuit breaker location known and accessible |  |

Table 3: Dolphin timing equipment list.

Make electrical connections:

Insert wiring diagram here

| Connection No. | Item | From | To |
| --- | --- | --- | --- |
| 1 | Timing laptop power supply | Power strip | Timing laptop |
| 2 | Wireless mouse USB adapter |  | Timing laptop |
| 3 | Timing printer power cord | Power strip | Timing printer |
| 4 | Timing printer USB cable | Timing printer | Timing laptop |
| 5 | Start system power supply | Power strip | Starting system External Power jack |
| 6 | Start microphone with start switch |  | Starting system Starter jack |
| 7 | Scoreboard power cord |  | Scoreboard |
| 8 | Dolphin base unit USB cable | Dolphin base unit | Timing laptop |
| 9 | Mini-BNC to BNC cable and BNC to dual banana adapter | Start system Start jack | Dolphin starter unit |
| 10 | Dolphin Scoreboard adapter |  | Scoreboard |
| 11 | Extension cord 1 | AC outlet #1 | Power strip |
| 12 | Extension cord 2 | AC outlet #2 | Scoreboard power cord |
| 13 | Ethernet cable | Timing laptop | Computer room router or switch |

Table 4: Dolphin electrical connection list.

Turn on equipment:

- Timing laptop

- Wireless mouse

- Timing printer

- Start system

- Scoreboard

- 24 Dolphin wireless stopwatches, by pressing the reset button for 5 seconds

Prepare for an outdoor environment if necessary:

- Clean timing laptop display with glass cleaner if dirty.

- Bring a sun shade to see the timing laptop display if the sun is at the wrong angle.

- Use paperweights to contain papers if windy.

- Bring flashlight if meet goes in to the night without adequate lighting.

Configure printer:

- On timing laptop, launch Printers and Scanners.  Make sure connected printer is listed as default.  If not:

- Select connected printer

- Click Manage

- Click Set as Default

- Click X to close window

Configure Dolphin:

- On timing laptop, launch Dolphin

- Verify all 24 watches and starter unit all have good reception and enough battery charge.  Swap out batteries and watches and reconfigure watches as needed.  To reassign a lane and position to a watch:

  - Turn the watch on by pressing and holding  the Reset button.

  - Enter the menu by pressing and holding the Reset button at the same time as either of the side buttons.

  - Once in the menu, the first setting is LANE. Press a side button to change the lane assignment for this watch.

  - Press Reset quickly to advance to the TIMER setting.

  - Press a side button to change the TIMER setting for this watch - A, B or C.  Must be uniquely set.

  - Press Reset quickly to advance to the MAN setting.

  - Press a side button to toggle to “off”, depending on a starting system to start the race.

  - Press a side button to select the channel number. This must match the channel on the starter and the base.

  - Press Reset quickly to exit the menu.

- Click Logging Enabled.  This will write the results to file and automatically advance the heat.

- Click Enable Scoreboard

- Set scoreboard lines = 1

- Set number of lanes to match pool configuration, i.e. 6, 7 or 8

- Set scroll time per referee preference, typically 2 seconds

- List by place then lane

- Click OK

- Make sure scoreboard is on.

Test start system:

- Ask the referee to announce and perform a test start.

- Verify loudspeakers are working for both voice and beep.

- Verify clock starts running.

Test wireless stopwatches:

- Press a black button on each stopwatch once.

- Verify all stopwatch times were recorded, like this:

Figure 32: Example Dolphin wireless stopwatch test results.

Test timing printer:

- Press Ctrl and PrnSc.  This should bring up Gadwin Printscreen capture box.

- If the box is not aligned with the Dolphin window, drag and drop the box edges to match.

- Press Enter to save box alignment changes.

- Press Enter to print.  Verify screen capture of window only is printed portrait to the timing printer.

### Timing Equipment Operator Duties

This person operates the laptop.  Their primary tasks are to capture times, save them both electronically and on paper, and record the file number associated with each heat and any changes in lane assignments.  Since it is easy to lose time data and the outcome can be undesirable, this person needs to do the following:

Either be already trained, or be trained on the job by an experienced operator.

Focus on collecting and recording data, not partaking in distracting activities such as having deep conversations with other people, or using a cell phone.

Keep track of which event and heat is in the water, and keep the Dolphin software heat and lane configuration updated accordingly.

Keep track of the last athlete swimming in a race, not saving the data before that last swimmer is in.

Print the screen and save the data quickly after a race to prepare for the next race to keep the meet on schedule, but…

Communicate clearly to the starting referee if there are problems, telling them firmly that they need to wait.

At the beginning of each event:

Write down the event start time on the timeline for referee reference.

Write down the event start time in the upper right corner of the Colorado page.

During each heat:

Write the file number in the left margin of the event program sheet stamped “Colorado”.  This is so the computer operators know which file to import results from.

Set the heat and lane information in the Dolphin software, making sure it matches what the Colorado sheet says.  The heat automatically advances in the software since data file logging is enabled, but if it’s a new event, or if the boys and girls heats are being run alternated across two events, you will need to edit both event and heat.

Cross out no-shows on the event program sheet with a pencil, and write DNS on the side.

Edit lane assignments as instructed by the referee with a pencil.

If a swimmer does not finish the race, write DNF next to their name with a pencil.

Add picture(s) of marked up Dolphin sheet – DNS, move swimmer, DNF, file names, event start time

After each heat:

Make sure all swimmers have completed the race.  Note: failure to do so can result in losing the time for that swimmer.

Click on Dolphin window to select it, if it is not already selected.

Press Left Ctrl-PrintScr.  This launches Gadwin Printscreen and shows a box where the capture window is.

If the capture window box does not line up with the Dolphin window, drag and drop the handles along the edges of the box until they do.  This should only have to be done once, unless someone moves the Dolphin window.

Press Enter.  Verify the printer prints the capture box.

Click “Reset Timers”.

Click “Yes” to confirm.  This saves the data to the file that the computer operators will import.

After each event:

Give the meet program stamped “Colorado” to the Chief Timing Officer.

Edit event in the Dolphin software to increment to next event.

After each session:

Wait until the results files are uploaded to the computer room.

Turn off computer, even if there is a session the next day.  This is done to make sure software doesn’t get in a stale state due to a memory leak.

### Intermediary Duties

After each heat:

Remove heat screen shot printout from the printer and verify the event and heat are correct.  If they are not, write the correct heat and event information in pencil on the printout.

Check each lane for 3 watch times.  If there is a missing time, circle the blank time in pencil to flag this discrepancy to the admin per the example below.

Figure 33: Example Dolphin results printout with missing time.

Check each lane for a span of times of less than 0.3 seconds.  If there is more than 0.3 second span, circle the 3 numbers in pencil to flag this discrepancy to the admin.

Figure 34: Example Dolphin results printout with more than .3 second span.

Compare order of finish on timing system printout to referee sheet.

If no lanes have a discrepancy, put a check mark in the right margin to indicate that you have looked at it.

Circle event and heat so it is easy to find.

After each event:

Collect the meet program stamped “Colorado” from the Timing Equipment Operator.

Collect the 2 to 3 meet programs stamped “Referee” and any DQ slips from the referees.

Staple DQ slips to respective green DQ cards.

Collect the 8 lane timer sheets from the lane timers via the Runner.  Make sure the event is correct and put them in numerical order.

Create an event packet with the paperwork clothes-pinned together in the following order:

DQ slips

“Colorado” meet program

“Referee” meet programs

Heat screen shots from Dolphin system, in heat order

Lane timer sheets, in lane order

Give event packet to Admin.

Reconfigure a spare timer for lane and seat

If a watch stops working, configure spare watch to match the correct lane and seat:

Press Reset and Stop buttons simultaneously for a few seconds

Press Reset to advance to next screen

Press Stop button to change displayed information

Verify channel is correct, matching other working watches i.e. “7”

Set lane to desired lane, i.e. “4”

Set seat to desired seat i.e. “C”

### Troubleshooting

If the Dolphin isn’t saving result files, short term use the screen shot printouts or files in a folder on the desktop; mid term switch it out with seeding or results computer but make sure and move the database file; long term may need reinstallation of the Dolphin software to fix it, TBD.  This may have been caused by a Windows Update but we are not sure.

If you can’t find the last Dolphin results file of the session, go press reset on the Dolphin computer to save it.  If they already shut down the app or the computer, you will have to manually enter the results from the printout.

If the scoreboard stops updating, power cycle the wireless scoreboard receiver.

If the printer stops printing, power cycle it.  If it still doesn’t print, try a different USB port.

If the scoreboard gets started before the referee starts it, don’t worry, one of the lane timers accidentally hit their button which started it, but the referee starter signal will re-start it.  The author sees this as a shortcoming of the system to not be able to disable starting from the lane timers.

If the Dolphin software stops writing files, power cycle and try again.  If it persists, swap computers and use one of the computer room computers for the Dolphin and vice versa – they are configured to be interchangeable.  Then after the meet take the Dolphin computer and uninstall the software, make a backup of the data files in C:\CTSDolphin, then delete the C:\CTSDolphin folder, then reinstall the software.  Something in the software seemed to get corrupted when this happened.

If you need to reprint a screen shot, you can find the screen shots saved in the My Computer \Pictures\My Screen Shots\ folder.  You can browse them until you find the file based on date and time and checking for the correct event and heat in the screen capture.

If the Dolphin program starts acting erratically -- such as resetting without the timing equipment operator initiating a reset, or not responding to the starter -- reboot the computer and relaunch Dolphin and set up logging and the scoreboard.  This could be caused by not turning off the Dolphin software or computer overnight and the software could have a memory leak that causes problems after several hours, resulting in unexpected behavior.

Raise base up high to get better reception from watches further away.

Type on keyboard slowly to not outrun the software lag.

Check battery levels at first boot of watches for best accuracy.  Replace if half full.

## System 6

Also applies to System 5.

### Equipment Setup

Equipment list:

| Item no. | Item | Note |
| --- | --- | --- |
| 1 | System 6 | Colorado Time Systems With timing harness, start, serial communications and parallel printer ports |
| 2 | System 6 power supply |  |
| 3 | Timing printer | Parallel interface with toner and white printer paper installed |
| 4 | Timing printer power cord |  |
| 5 | Timing printer parallel cable |  |
| 6 | Start system | Includes box with speaker, light, start in and dual BNC start out ports mounted on tripod |
| 7 | Start system power supply |  |
| 8 | Start microphone with start switch |  |
| 9 | 8 lane scoreboard | No names |
| 10 | Scoreboard power cord |  |
| 11 | Primary harness | For start, touch pads and 1 button |
| 12 | Backup harness | For buttons 2 and 3, if used |
| 13 | Starter cable |  |
| 14 | Scoreboard cable |  |
| 15-22 | 8 touch pads |  |
| 23-46 | 24 buttons |  |
| 47 | Power strip | At least 3 outlets for System 6, timing printer and starting system |
| 48 | Extension cord #1 | For power strip |
| 49 | Extension cord #2 | For scoreboard |
| 50 | Serial cable | Long enough to reach from System 6 to computer room computer USB to serial adapter |
| 51, 52 | External speakers |  |
| 53 | Reliable power source with circuit breaker location known and accessible |  |

Table 5: System 6 timing equipment list.

Make electrical connections:

Cable harnesses

from start system, touch pads and buttons

Serial cable

to Computer room

Printer

Championship Start System

Power Strip

Microphone and Start Switch

System 6 power supply

From AC Outlet

Back of System 6

43

4

1

2

5

7

9

10

45

External speakers

Scoreboard cable

to scoreboard

46

8

3

Start system power supply

Start cable

Figure 35: System 6 electrical connection diagram.

| Connection No. | Item | From | To |
| --- | --- | --- | --- |
| 1 | System 6 power supply | Power strip | System 6 External Power jack |
| 2 | Timing printer power cord | Power strip | Timing printer |
| 3 | Timing printer parallel cable | Timing printer | System 6 Printer Port |
| 4 | Start system power supply | Power strip | Starting system External Power jack |
| 5 | Start microphone with start switch |  | Starting system Starter Jack |
| 6 | Scoreboard power cord (not shown) |  | Scoreboard |
| 7 | Start cable | Start system Start jack | Primary harness Start input |
| 8 | Scoreboard cable | System 6 SCBD jack | Scoreboard |
| 9 | Primary Harness |  | System 6 Primary Input |
| 10 | Backup Harness |  | System 6 Backup Input |
| 11-18 | Touch pads (not shown) |  | Primary Harness Prime inputs |
| 19-26 | A stop buttons (not shown) |  | Primary Harness A inputs |
| 27-42 | B and C stop buttons (not shown) |  | Backup Harness B and C inputs |
| 43 | Extension cord 1 | AC outlet #1 | Power strip |
| 44 | Extension cord 2 (not shown) | AC outlet #2 | Scoreboard power cord |
| 45 | Serial cable | System 6 | Computer room router or switch |
| 46 | External speaker cable | Start system Speaker Cable jack | External speakers |

Table 6: System 6 electrical connection list.

Turn on equipment:

- System 6

- Timing printer

- Start System

Prepare for an outdoor environment if necessary:

- Adjust contrast knob on back of System 6 for maximum brightness.

- Clean System 6 display with glass cleaner if dirty.

- Bring a sun shade to see the System 6 display if the sun is at the wrong angle.

- Use paperweights to contain papers if windy.

- Bring flashlight if meet goes in to the night without adequate lighting.

Configure System 6:

Display

Console

Keyboard Insert

Numeric Keypad

Figure 36: System 6 operator interface.

- Press “Sports Menu” then “Swimming” to get to the main menu.

- Press “Battery Check” on Console and make sure greater than two hours of running time are left on the battery, and that AC mode is on.

- Press “Setups” Soft Key on the Console.  Navigate within setup menu with up and down soft keys to select the group on the left, then with the numeric keypad to toggle or select a field for further configuration.

- Select “Start”.

  - Press “1” for “Automatic”.

- Select “Finish/Buttons”.

  - For Zone championship meets, the touch pads will be primary and there will be 3 backup buttons.  For these meets press “1” for “Pad (Prime Finish)” and press “7” for “Three Buttons (backup finish)”.

  - For SRJC meets, the touch pads will be primary and there will be 1 backup button.  For these meets press “1” for “Pad (Prime Finish)” and press “5” for “One Button (backup finish)”.

  - For meets where the touch pads are malfunctioning, 3 buttons may be used.  For these meets press “4” for “Three Buttons (Primary Finish).”

  - Press “8” for “Compare button early to pad” so soft touches are highlighted in the results for admin review.

- Select “Hardware Setup”:

  - Press “1” to select “Speaker Volume” then type “1”, “2” or “3” and then “Enter” to set level.  Usually 3 is best to hear the touch pad register beeps in a noisy indoor pool.

  - Press “3” to “Allow Remote Setup” from computer via serial port.

- Select “Splits”:

  - Press “1”to select “Cumulative” splits to be printed.

  - Press “2” to select “By Lap” splits to be printed.

- Select “Timing”:

  - Select “1” for .01 second accuracy or “2” for .001 second accuracy, depending on what the head referee wants, typically .01 seconds.

  - Press “3” and “15” then “Enter” to set near end pad split delay to 15 seconds.

  - We don’t use far end pads, so no need to set them up.

  - Press “6” and “15” then “Enter” to set pad delay after start to 15 seconds.  Note: default is 3 seconds; if you don’t change this, swimmers exiting the pool after non-relay flyover starts can trigger a false touch.

  - Do not press 5, 7 or 8 to warn about predicted times or count down laps, just pay attention instead.

- Select “Pool”:

  - Select “1” if the nearest lane to the Colorado is lane 1, or “2” if lane 1 is farthest from the Colorado.

  - Press “3” and enter 6, 8 or 10 lanes.  If running an odd number, select the next highest even number and then turn off the highest lane as described later.

  - Do not select “Far End Splits” since we never use them.

  - Press “5” for short course pool or “6” for long course pool.

  - Press “7” if yards or “8” if meters.

- Select “Scoreboard”:

  - Press “1” to select “high speed” data transmission.

- Select “Printer”:

  - Press “1” and select “IBM” as printer type.

  - Press 3 for “Store/Print Format”.  Here is where the navigation gets complicated.   Select “Race Summary” to print both by lane and by place, and then scroll down and select “Line Feed”.  Use soft keys to navigate and remove or add elements.  Use upper Enter key in Console to enter changes, not lower Enter in numeric keypad.

- Select “Event Sequence”:

  - No need to do anything here, since this will be set once the session is downloaded from the computer in to the System 6.

- Select “Time/Date”:

  - Set the “Time”, “Day” and “Date” if necessary.

  - Select 12/24 hour clock format as desired by Meet Referee – typically 12 hour.

- Press “Record Setups” Soft Key to write preferences to non-volatile memory that will persist across power cycles.

- Press “yes” when it asks you again.

- Press “Quit” to exit setup menu.

- Press “Scoreboard” Soft Key.

  - Press “Scoreboard On” Soft Key.

  - Press “Scoreboard Blank” and verify time of day is displayed on the Scoreboard.

  - Press “Scoreboard Blank” again to blank the scoreboard.

  - Press “Quit”.

- Press “Display” Soft Key.  Verify window on display comes up with lane vs. pad and button pressed table.

- Insert reset display screen shot here

Test start system:

- Press “Edit Event/Heat” button on Keyboard Insert.

- For testing purposes only, change Event Number to 99, Heat Number to 1, and Events length to 50 and press Enter.

- Ask the referee to announce and perform a test start.

- Verify loudspeakers are working for both voice and beep.

- Verify clock starts running.

Test touch pads and buttons:

- Press all touch pads and backup buttons.

- Verify all touches and buttons were recorded, like this:

Insert completed display screen shot here

  - If touch pads are shorting out and falsely indicating touches, remove pad from water and use touch pad pump needle to let air in.

  - If touch pads are not recording presses, remove pad from water and use touch pad pump to pull air out of the pad.

Test timing printer:

- Press Store/Print and verify printer prints properly.  Output should look like this:

Replace picture with non-marked up System 6 results printout

Figure 37: Example System 6 race results printout.

### Timing Equipment Operator Duties

This person sits in front of the Colorado which is situated behind the starting Referees. This person’s job is to:

- Manually synchronize the Colorado with the race event and heat numbers as decided by

the Starter Referee and then arm the Start process

- Visually verify timer is started, and split and finish data is being collected by the machine; troubleshoot the system if it is not working properly

- Manually invoke the saving of data for a valid race and race summary printout process

- Collect Colorado Meet Sheets from the Runner, and annotate them with race number,

no shows and DQs submitted during an event

- Collect DQ slips and Referee Meet Sheets from Referees

- Give Colorado and Referee Meet Sheets and DQ slips to Chief Timing Officer

Perform the following sequence for each heat:

After data has been stored and printed from a previous heat, press both “Reset” buttons simultaneously.  Verify display then shows “Reset”.

NOTE 1: YOU WILL LOSE DATA IF YOU DO NOT STORE/PRINT BEFORE RESET!

Note 2: if there is a start after Store/Print but before Reset, the Colorado will ask “Start detected, valid start?” and if it is, press yes.  DO NOT PRESS RESET!

If the next race is the first heat of a new event, press “Next Event” on the Keyboard Insert.  If an event number is being skipped because boys or girls are not doing the corresponding race, press “Next Event” again.  Otherwise press “Next Heat”.  Verify Event and Heat on Display are correct, compared to meet sheet and what the starter is announcing.

Note: if you forget to update the heat and event before the start of the race, you can do it before the end of the race, but you can’t do it after Store/Print.

If any lanes are not occupied, either per the meet sheet or due to a no show, deselect them by pressing the corresponding “Lane On/Off” button on the Keyboard Insert.  Make sure every lane that is not vacant is turned on and was not left off from a previous heat lane vacancy.  If the vacancy is due to a no show, cross out the swimmer on the Colorado Meet Sheet and write “No Show”.

You may want to indicate to the Starter when you are ready.  Then be quiet while they start the race.  Verify that the clock on the display starts to advance and the printer prints a few lines once the starter beep and strobe goes off.

Record the last three digits of the race number on the Display on the left hand margin of the Colorado Meet Sheet with a red pen.

Assuming touch pads are being used and hence splits are being taken:

- Verify “S Armed” comes up after 15 seconds for each lane for longer races with 1

or more splits, 	or “F Armed” comes up for shorter races with no splits.  Note that there is a delay so a swimmer from the previous race could get out of the pool and press the touch pad without accidentally counting a split.

- If a split pad touch does not get pressed on a lane, which is common, press

“+ Touch” on Keyboard Insert and enter lane number on the Numeric Keypad.

- If you lose track of missed touches and you know the swimmer is coming in to the

finish, which is easy to do, press “Finish Arm” on their corresponding lane.

- If you mistakenly press “+ Touch”, which is less common, press “- Touch” on

Keyboard Insert and enter lane number on the Numeric Keypad.

I don’t know why one would ever press “Split Arm” on a given lane.

Watch the swimmers come in, listening for one beep per button or pad touch.  As soon as all the swimmers complete, verify all pads and buttons have been pushed on the Display.  If no presses occur, notify the Admin, so the lane timer stop watch time or a coach’s timer time can be used instead.  Determine why no buttons were pushed and rectify the situation.

Once all swimmers are in, press “Store/Print” button on Keyboard Insert.  Verify “Stored” message comes up with in a few seconds, then the printer starts printing and advancing the paper immediately.  Also reverify the race number is what you wrote on the meet program margin.

NOTE 1: RACE DATA WILL BE LOST IF “STORE/PRINT” IS NOT PRESSED BEFORE “RESET’!

NOTE 2: DO NOT TURN OFF COLORADO AT END OF MEET UNTIL COMPUTER OPERATOR HAS TOLD YOU THAT ALL RESULTS DATA HAVE BEEN RETRIEVED.

At the end of each event:

4.9 Pass Colorado program sheet to the Intermediary.

After the session ends:

If trials, power cycle Colorado and have computer operator load finals session in to Colorado.

After the Meet Ends:

6.  Shut Down

Press “Quit”

Press “Scoreboard”

Press “Scoreboard Blank” Twice.  Verify the Scoreboard goes off.

Press “Quit”.

At the end of the day, check with the Meet Manager Computer Controller to make sure they have remotely stored all data inside the Colorado to the Computer before powering down the primary Colorado or disconnecting the DB9 cable from its COM1 port.

NOTE: TURNING OFF THE COLORADO BEFORE THE COMPUTER ROOM DOWNLOADS THE DATA COULD RESULT IN LOSS OF DATA!

Turn off all equipment.

If the meet is over or equipment is not secure or is exposed to weather, disconnect all equipment and put it inside.

7. Troubleshooting

If buttons or pads seem to be intermittent, reseat connectors at lanes and in to the back of the Colorado.

If pads are too hard to press, evacuate air from bladder with pump and needle.

If pads are shorting, inject air in to bladder with pump and needle.

If comm test passes but meet does not download over a long serial cable, bring computer next to Colorado and connect them with a short serial cable and retry.

### Intermediary Duties

This person sits between the Timing Equipment Operator and the Admin or head Referee.

This person’s job is to:

- Verify printer is working properly; troubleshoot and fix any printing problems that may

Occur while the timing equipment operator concentrates on capturing times

- Collect the Race Summary Sheets from the printer

- Verify buttons worked and were pressed in a timely fashion, annotate the race

summary sheet accordingly, and work with the admin or head referee when there

is a problem

- Collect Colorado and Referee Meet Sheets and DQ slips from the Automatic Equipment Operator

- Collect DQ slips from Referees after the event and annotate the Colorado Meet Sheet

- Collect Lane Timing Sheets from the Runner

- Clip together the DQ slips, Colorado and Referee Meet Sheets, Race Summary Sheets

and Lane Timing Sheets for an event and give to Runner

The following sequence will be performed for each race:

Collect Race Summary Sheet from the printer.

Circle the event number and heat number with a pencil.

Verify at least one button or pad was pressed and the times did not span more than 0.3 seconds for each lane, and that it selected the middle time as the Finish Time.  If so, draw a line next to the lane time column with a pencil indicating you have checked it and it is good.  If not, circle a single time if it is a single outlier or is missing, or circle the whole column if all the numbers are far from each other or all missing, to flag admin or head referee that there is a problem.

If more than one button or pad was pushed and the span as greater than 0.3 seconds, verify that the Colorado threw out the outlying data points.

- If one time was an outlier, make sure Colorado picked the time of the two

other times that was closest to the outlier for the Finish Time.

- If all three data points were spread far apart, make sure the Colorado picked the middle time for the final time.

If only one button was pushed, make sure the Colorado selected it for the Finish Time.

If there were no buttons pushed, or there was a problem with the timing system, notify the head Referee so they can look at the Lane Time Sheet or Coach’s Timer to determine the Finish Time.

If a button or pad is regularly not being pressed but it appears like someone is trying to press it, check the finish cable harness seating and switch connection seating, or send someone over to talk to the timer and possibly swap out their button or a pad.  Note that this could cause a delay in the meet.

If anyone was DQd after the event was over, cross them out on the Colorado Meet Sheet and write DQ with a red pen, tear off the carbon copy of the DQ form, and hand it to the Admin or Chief Stroke and Turn Judge.

Compare order of finish on timing system printout to referee sheet.

Figure 38: Example System 6 marked up heat results printout.

The following sequence will be followed at the end of each event.

When event is over, clip the paperwork together in the following order:

- DQ slips

- Colorado meet sheets

- Referee meet sheets

- Lane timer sheets

Give the stack of paperwork to the Runner who will bring it to the Computer Room.

This page intentionally left blank.

## Appendix

### Quantum Timing Equipment Operator Checklist

Before Each heat:

- Update start list for deck seeded meets only.

During each heat:

- Select + if there is a soft touch.

- Select – if there is an extra touch.

- Select Arm touchpad if you missed one or more soft touches and are certain the swimmer is coming in for the finish.

- Select DNS if a swimmer does not show.

- Select DNF is swimmer does not finish.

- Select Take backup time if there were too many or too few touches and corrections.

- Toggle Used/Unused for swimmers that scratch before the heat.

After each heat:

- When all swimmers are finished, press Officialize thumbs up button.

- Select next heat/event.

After each event:

- Give timing equipment operator program sheet to intermediary.

This page intentionally left blank.

### Dolphin Timing Equipment Operator Checklist

During each heat:

- Verify event number and heat number are correct.

- Write file number on left margin of Colorado meet program sheet.

After each heat:

- When all swimmers are finished, press Left Ctrl-PrntScrn.  Verify Gadwin Printscreen box is around Dolphin window.

- Press Enter to print screen and save to file.

- Type “r” for Reset.

- Type “y” to confirm.

After each event:

- Give timing equipment operator program sheet to intermediary.

This page intentionally left blank.

### Colorado Timing Equipment Operator Checklist

During each heat:

- Turn off unused lanes.

- Verify event number and heat number are correct.

- Write file number on left margin of Colorado meet program sheet.

- Press + if there is a soft touch.

- Press – if there is an extra touch.

- Press Finish Arm if you missed one or more soft touches and are certain the swimmer is coming in for the finish.

After each heat:

- When all swimmers are finished, press Store/Print.

- Press Reset with two fingers.

After each event:

- Give timing equipment operator program sheet to intermediary.

This page intentionally left blank.

### Intermediary Checklist

After each heat, if results being printed:

- Collect timing equipment heat result printouts from printer.

- Circle heat and event.

- Circle any missing watch times.

- Check all button and pad times across a lane to make sure they do not vary more than 0.3 seconds.  If they do, or if there is a late touch flagged, circle the lane row.

After each event:

- Create a packet of event papers held by a clothespin with the papers in the following order:

    - DQ slips from Chief Stroke and Turn Judge.

    - Meet program from timing equipment operator.

    - Referee meet program(s).

    - Timing system heat result printouts, if being printed.

    - Lane timer sheets delivered by runner.

- Give event packet to admin or meet referee for review before sending it to the computer room.